CCS6224-NETWORK SECURITY

Assignment (30%)

Term: 2430

Group Name: Hawkeye

Title: Network Security Topology Design

| ID | Name | Contribution (in %) |
| --- | --- | --- |
| 1211103319 | SATHISHKUGAN A/L THANGARAJOO | 25% |
| 1211103357 | SHARVIN A/L GUNALAN | 25% |
| 1221304358 | NITYA A/P SATHU OM | 25% |
| 1211104170 | THASAYAINI A/P MURTHY | 25% |
|  | Total | 100 |

# Security Policy Documentation

1. Password Policies:

- Enable secret password: hawkeye
Ensures encrypted storage of the privileged EXEC mode password to prevent unauthorized access.

- Console and VTY password: hawkeye
Secures console and remote access to the device by requiring authentication before granting access.

- Service password encryption enabled
Encrypts all passwords stored in the device configuration to prevent them from being viewed in plaintext.

- Local user authentication:

  - Username: avenger

  - Secret: hawkeye
Creates a local user account for authentication and provides an additional layer of security.

2. Banner Message of the Day (MOTD):

-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-

UNAUTHORIZED ACCESS IS PROHIBITED

-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-

Displays a warning message to all users attempting to access the device, deterring unauthorized access and informing them of legal consequences.

3. SSH Configuration:

- Domain name: hawkeye.com
Required for generating cryptographic keys used in SSH encryption.

- RSA key generated with 1024-bit encryption
Provides a secure encrypted communication channel between remote devices and the network device.

- SSH version 2 enabled
Ensures a more secure and robust connection compared to SSH version 1.

- VTY lines configured for SSH-only access with local authentication
Restricts remote access to SSH, preventing less secure protocols like Telnet from being used.

4. IP NAT Configuration:

- Inside NAT applied to interfaces G0/0, G0/1, and G0/2
Translates private IP addresses to public IP addresses for internet access.

- Access list 50 permits NAT for 192.168.20.0/24
Defines which internal IP addresses are allowed to be translated for outbound traffic.

- NAT overload applied to G0/2 interface
Enables multiple private IPs to share a single public IP using port-based translation (PAT), optimizing IP usage.

5. Access Control Lists (ACLs):

- ACL 10:

  - Permit 192.168.20.192/26
Allows access only to specific internal network segments.

  - Deny all other traffic
Blocks all traffic that is not explicitly permitted, enforcing security controls.

  - Applied to VTY lines for restricted remote access
Ensures only authorized users from specific IP ranges can access the device remotely.

- Extended ACL RES-ACCESS:

  - Permit ICMP traffic
Allows diagnostic tools like ping and traceroute for network troubleshooting.

  - Permit UDP traffic on ports 67, 68 (DHCP)
Enables DHCP communication, allowing automatic IP address assignment.

  - Permit UDP and TCP traffic on port 53 (DNS)
Allows domain name resolution for accessing websites and services.

  - Permit TCP traffic on ports 80 (HTTP), 25 (SMTP), 20 (FTP Data), 21 (FTP Control)
Ensures web browsing, email communication, and file transfers function properly.

  - Permit UDP traffic on ports 50, 450
Allows specific application-based communication.

  - Applied inbound on interfaces: SERVER, ISP1, and ISP2
Restricts access to only necessary services at the network edge.

6. Firewall NAT Rules:

- HQ NAT Rules:

  - Defined object networks for each department (HR, FINANCE, SALES, IT, SERVER)
Separates network traffic based on department for better management and security.

  - Configured dynamic NAT for ISP1 and ISP2 for redundancy
Ensures internet connectivity is maintained even if one ISP fails.

  - Subnet definitions for 192.168.20.0/26, 192.168.20.64/26, 192.168.20.128/26, 192.168.20.192/26, 192.168.21.0/28
Organizes the network into well-defined segments, improving efficiency and security.

7. Network Security Recommendations:

- Regularly update passwords and encryption keys
Protects against unauthorized access by ensuring outdated credentials cannot be exploited.

- Implement multi-factor authentication for remote access
Adds an extra layer of security beyond passwords, reducing the risk of compromise.

- Monitor firewall logs and NAT translation tables for unusual activity
Detects and mitigates potential security threats in real time.

- Restrict SSH access to specific trusted IP ranges
Prevents unauthorized users from attempting remote logins.

- Review ACL policies periodically to ensure minimal privilege access
Reduces security risks by enforcing the principle of least privilege.

- Apply IDS/IPS solutions for advanced threat detection
Identifies and mitigates potential cyber threats before they can cause harm.

8. Backup and Disaster Recovery:

- Configuration saved using “wr mem” command
Ensures all changes to the device configuration are stored permanently.

- Regular configuration backups stored in a secure location
Provides a recovery option in case of device failure or cyberattack.

- Implement redundancy for ISP links using load balancing
Ensures continuous network connectivity by distributing traffic across multiple ISPs.

9. Compliance and Audit:

- All security policies comply with industry best practices
Ensures the network remains secure and aligned with security standards.

- Regular audits to ensure adherence to security configurations
Identifies and fixes vulnerabilities through periodic reviews.

- Documentation updated after every major network change
Maintains an up-to-date record of all security policies and configurations.